Building and Testing a WiFi BFI Person-Identification + Attendance PoC on the Cheap 802.11ac Route

TL;DR

- It works on cheap 802.11ac, with real caveats. You can reproduce the core of BFId (KIT, CCS 2025) using the open-source Wi-BFI tool and an inexpensive USB adapter, but the single most important decision is the chipset: use a MediaTek MT7612U adapter (Alfa AWUS036ACM ~$37–48, or a generic "no-name" MT7612U ~$12–25), NOT the popular Realtek RTL8812AU (Alfa AWUS036ACH / ASUS USB-AC56), which is documented to corrupt/drop multi-stream 802.11ac VHT captures. Capture is fully passive — you sniff existing beamforming feedback frames and never need the WiFi password.

- Going cheap (5 GHz / 80 MHz) vs the paper's 6 GHz / 160 MHz costs you spatial resolution, not the concept. The paper's 160 MHz setup produced 740 BFI features (10 quantized angles × 74 sub-channels) per time point; an 80 MHz 802.11ac channel sounds ~234 sub-channels, roughly halving the feature dimensionality and lowering the accuracy ceiling and robustness. But the same Restuccia-group hardware (802.11ac / 80 MHz) already hit up to 99.28% activity-recognition accuracy with Wi-BFI, so the identification concept remains viable at reduced scale.

- BFI and RADIUS are complementary; BFI's real product value is presence assurance and anti-spoofing, not identity. Network login (RADIUS) gives deterministic identity but only proves a device is connected; BFI gives a probabilistic biometric that confirms a real, moving body is physically present. Fuse them at the decision/score level: RADIUS asserts "who," BFI confirms "actually here." Expect the 99.5% lab accuracy to degrade substantially in a real office with simultaneous movers, clothing changes, and environmental drift.

Key Findings

The two anchor papers

- BFId (Julian Todt, Felix Morsbach, Thorsten Strufe; KIT / KASTEL Security Research Labs; ACM CCS 2025, DOI 10.1145/3719027.3765062) identified individuals from a user study of 197 participants (usable subsets: 161 for BFI, 170 for CSI, due to technical unreliability) at 99.5% ± 0.38 accuracy using BFI. Verified core claims: BFI is broadcast unencrypted on the MAC layer and capturable passively with commodity hardware without network access; a single sniffer records multiple perspectives simultaneously; identification collapses across mismatched perspectives (all <5% when trained on perspective 1; up to ~25% for the two most similar perspectives); empty-room recordings could not be matched to people (2.34% ± 0.64 top-2), proving it senses body biometrics rather than session artifacts; BFI outperformed CSI (CSI reached only 82.4% on the full dataset with the same simple model); BFI stayed above 90% with only 20% training data (CSI fell to ~28%); and BFI is robust to lower sample rates (time-coarsening had only a small effect).

- Wi-BFI (Khandaker Foysal Haque, Francesca Meneghello, Francesco Restuccia; Northeastern / Univ. of Padova; ACM WiNTECH 2023, DOI 10.1145/3615453.3616514) is the open-source Python tool (github.com/kfoysalhaque/Wi-BFI) that captures and decodes BFI from both 802.11ac and 802.11ax at 20/40/80/160 MHz using a WiFi card in monitor mode with Pyshark / tshark / Wireshark. It reconstructs the compressed beamforming angles (φ, ψ) and the V-matrices. The older MATLAB tool (github.com/kfoysalhaque/MU-MIMO-Beamforming-Feedback-Extraction-IEEE802.11ac) targets 802.11ac MU-MIMO specifically. Wi-BFI's own use case demonstrated human activity recognition "achieving up to 99.28% recognition accuracy" on 802.11ac / 80 MHz.

Hardware — the cheap 802.11ac route (the critical decision)

- Sniffer adapter: use MediaTek MT7612U; avoid Realtek RTL8812AU for capture fidelity.

  - The RTL8812AU (Alfa AWUS036ACH, ASUS USB-AC56, COMFAST CF-912AC) is dual-band 802.11ac and technically supports monitor mode, but is documented to corrupt or drop multi-spatial-stream VHT frames. The WLAN Pi project's investigation of the RTL8812AU-based CF-912AC found that "802.11ac data frames that were sent using 2 spatial streams were not being captured … single stream 802.11ac data frames were captured OK," attributed to the closed Realtek driver. An SNBForums report using an RTL8812AU (ASUS USB-AC56) stated the capturing device could see the AP's VHT feedback request but "the responses from a non-capture STA are corrupt … it's the PCAP from the driver." Because BFI/BFA report frames are the exact thing you need clean, this is a real risk.

  - The MT7612U (Alfa AWUS036ACM ~$37–48; generic "no-name" MT7612U ~$12–25) is the community-validated reliable choice. The WLAN Pi team found "the MediaTek MT7612U driver seems to perform very well and has no issue with the 2-stream over-the-air capture. The Mediatek MT7612U adapter option has become a favourite among the developer team." It uses the native in-kernel mt76 driver, supported since Linux kernel v4.19 (2018) — no compilation, plug-and-play on Ubuntu/Kali/Raspberry Pi. Its limitation: it is a 2×2 / 80 MHz device — it cannot capture 3+ spatial-stream VHT, 160 MHz, or any 802.11ax. That is acceptable for 802.11ac beamforming feedback work but is a hard ceiling if you later need 160 MHz or WiFi 6/6E.

  - Caveat on frame type: the RTL8812AU corruption is worst for high-rate, multi-stream VHT-modulated data frames. The BFA/compressed-beamforming action frames are sent at legacy management rates, so RTL8812AU may partially capture them — but one direct test reported corrupt VHT feedback, so the safe, cheap pick is MT7612U.

  - The Wi-BFI / BeamSense authors do not publish the exact sniffer NIC they used; they state that "any IEEE 802.11ac-compliant NIC set in monitor mode could be used." The Netgear Nighthawk X4S routers in their papers are the AP/STA infrastructure, not the sniffer.

- Access point requirements: You need an 802.11ac AP that performs beamforming (ideally MU-MIMO). The Restuccia group used Netgear Nighthawk X4S AC2600 routers as beamformer and beamformees (BeamSense: 3×3 MU-MIMO, one AP with M=3 antennas and three single-antenna STAs, channel 153, 80 MHz). MU-MIMO requires 802.11ac Wave 2 APs and compatible clients; SU-MIMO beamforming is more widely supported. You do not need to control the AP to sniff passively — but for a controlled PoC you should deploy your own AP + a client (beamformee) + generated traffic (e.g., iperf3) so that channel sounding is triggered frequently. BFI only exists when the AP triggers sounding, which is traffic-dependent; the paper tuned traffic (200 Mb/s TCP for BFI) to maximize sounding rate, achieving ~10 Hz BFI sample rate.

- Software / OS: Linux (Ubuntu or Kali). Install tshark, wireshark, aircrack-ng. For Wi-BFI, create the conda environment (conda env create -f wi-bfi.yml). The MU-MIMO extraction tool additionally requires MATLAB.

The cheap-vs-6E difference and its effect on results

- Why the paper chose 6E: They used two TP-Link Archer BE800 APs on 6 GHz channels 37 and 85 (160 MHz-wide, WiFi 6E) to minimize interference from other WiFi devices and maximize sub-channel count, with Intel AX210 NICs (the only option supporting both 6 GHz and CSI extraction). Their 160 MHz BFI produced 740 features (10 angles × 74 sub-channels) vs 212 for CSI (magnitude + phase for 53 subcarriers × 2 antennas).

- What you lose going to 802.11ac / 5 GHz / 80 MHz: An 80 MHz channel sounds far fewer sub-channels. Per the Restuccia group's 802.11ac testbed, an 80 MHz channel "allows sounding K=234 sub-channels, i.e., 256 available sub-channels on 80 MHz channels minus 14 control sub-channels and 8 pilots." Fewer sub-channels = fewer BFI feature dimensions = lower spatial resolution and a lower accuracy ceiling, plus more interference/congestion on the crowded 5 GHz band. The paper explicitly attributes BFI's edge over CSI partly to the higher spatial resolution reflected in the larger feature count, so halving your sub-channels is the main realistic penalty.

- What you keep: The paper notes the band itself matters little — prior work found only small differences between 2.4 and 5 GHz, and they expect the 6-vs-5 GHz difference to be even smaller. So the penalty is channel width, not frequency. The concept remains valid on 11ac: Wi-BFI achieved up to 99.28% activity recognition, and its average over all setups using all 234 sub-channels at 80 MHz was 99%. Identification (harder than activity recognition, especially at scale) will land below the 6E ceiling, but a convincing PoC is achievable.

Controlled setup and enrollment protocol

- Paper's physical layout: Two APs on separate channels (one for CSI traffic, one for BFI). Four capture perspectives: (1) parallel to the line-of-sight (LOS), (2) across-LOS mounted high at 210 cm, (3) across-LOS at hip height (the most common perspective in prior work), and (4) non-LOS in a neighboring room. APs and antennas mounted at (or slightly above) hip height to best capture gait (except the deliberately-high perspective 2). For each perspective, two antennas were mounted orthogonally — one pointing up, one orthogonal to the LOS.

- Enrollment per person: Participants walked a fixed path in five styles — normal (repeated 20× back-and-forth), plus backpack, carrying a crate, through a turnstile, and fast (10× each). This produced ~40 normal BFI walking sequences per person (average sequence length ~40 samples at ~10 Hz), ~6,312 normal BFI sequences across 161 people. To keep gait unobstructed, participants wore no baggy clothes, skirts, dresses, or heeled shoes. Sessions took up to an hour (participants paid €15). Labels came from a side channel (e.g., the phone's MAC address in the adversary model).

- Minimal cheaper version / minimum people: The paper argues datasets under ~50 people are unconvincing for real-world claims (only one prior work exceeded 50). But for a first PoC, 5–15 people (matching most prior CSI literature) is enough to prove the concept, with a single fixed perspective and just the normal walking style, then scale up.

- Perspective-dependence — the key deployment implication: Identification collapses when the capture geometry differs between enrollment and identification (cross-perspective accuracy <5%). Therefore sensor geometry must be fixed, and re-enrollment is required if the environment or sensor placement changes. This makes a fixed choke-point (doorway/turnstile) far more realistic than free movement in an open room.

The ML model

- Architecture: Feature standardization only (no bandpass filtering, FFTs, or PCA) → a standard LSTM → two fully-connected layers, each with batch normalization and ReLU → a softmax classification layer. Trained with the ADAM optimizer until convergence. The model handles variable-length inputs (walking speed differs per person and per trip) and adds one extra feature per timestep: the time elapsed since the last data point. Hyperparameters (initial learning rate, batch size, weight decay, LSTM and FC layer sizes) were tuned with the Tree-structured Parzen Estimator via Optuna. Evaluation used an 80:20 train/test split, repeated 5× with independent splits, reporting identification accuracy (correct classifications / total).

- Inputs & implementation: BFI input dimension = 740 features in the paper's 160 MHz setup; on 80 MHz expect roughly half (scale with sub-channel count). Implement in PyTorch (the more natural fit for a custom variable-length LSTM) or TensorFlow. The design is deliberately simple — the authors treat it as a lower bound on achievable accuracy, meaning purpose-built preprocessing and architectures could do better.

Integration with RADIUS-based attendance software (the full-system concept)

- RADIUS side (deterministic identity): FreeRADIUS accounting emits Accounting-Request records with Acct-Status-Type = Start / Interim-Update / Stop, stored in the radacct table keyed by username and session ID, with connect/disconnect timestamps. This deterministically proves a device belonging to user X authenticated and when. Simultaneous-Use can prevent double logins. This is your certain "who" and "when connected."

- BFI side (probabilistic biometric presence): The Wi-BFI pipeline extracts BFI angle time-series → the LSTM classifier outputs a per-person probability (softmax) that a real, moving body matching an enrolled person is in the room.

- Where BFI adds value over login alone: (1) Anti-spoofing / left-behind-device detection — RADIUS says the laptop is connected, but BFI confirms whether the human is actually physically present and moving, defeating "I left my laptop logged in to fake attendance"; (2) a presence-confidence score you can log and threshold; (3) potential device-free presence even when the phone is off/absent.

- How to fuse: Use decision/score-level fusion, the standard practical choice in the multimodal-biometrics literature (score-level fusion is favored for ease of implementation and good performance). Concretely: on a RADIUS Start for user X, open an "expected present" window and mark attendance "device-only"; if the BFI classifier's confidence that user X's body is in the room exceeds a threshold within that window, upgrade to "confirmed present." RADIUS supplies the closed identity set (you only need to distinguish among the handful of people the network says are logged in — a much easier classification problem than open-set), and BFI supplies physical-presence assurance.

Realistic real-world limitations

- Co-presence interference (the biggest one): WiFi gait identification is fundamentally a single-person-at-a-time technique. When multiple people move simultaneously, their gait signatures superimpose in the signal and must be separated (an unsolved blind-source-separation problem on commodity hardware). Multi-person WiFi gait ID on commodity devices remains largely unproven; expect it to fail in a busy open office.

- Environmental drift & clothing: The paper's controlled clothing restrictions (no baggy clothes) likely overestimate accuracy; real clothing variety, plus furniture/layout changes and cross-day environmental drift, degrade performance and can require re-enrollment.

- Scaling: 197 people is an order of magnitude larger than prior work but still far below hundreds/thousands; accuracy trivially drops as the population grows. The paper could not test open-set generalization (softmax over a closed set), so it cannot identify people not in the training data.

- Realistic expectation: Outside the controlled lab, expect accuracy substantially below 99.5% — this is explicitly unproven and flagged by the authors as future work.

Details / Concrete Commands

BFI capture pipeline (802.11ac, MT7612U, Linux):

# 1. Enter monitor mode

sudo airmon-ng start wlan0

# 2. Tune to your AP's channel and 80 MHz width (e.g., ch 149)

sudo iw dev wlan0mon set channel 149 80MHz

#    (or by frequency: sudo iw wlan0mon set freq 5745 80 5775)

# 3. Capture beamforming feedback frames

sudo tcpdump -i wlan0mon -w capture.pcapng

#    In Wireshark, confirm you see VHT Compressed Beamforming action frames

#    (802.11 Action No Ack, VHT category); check wlan_radio.11ac.beamformed

# 4. Extract BFI with Wi-BFI

python main.py capture.pcapng AC MU 3x1 80 <beamformee_MAC> 200 V_out bfa_out

For the MATLAB MU-MIMO tool: install wireshark aircrack-ng tshark, capture as above, run Feedback_split_stations.sh (edit STA MAC addresses) to split per-station BFI via tshark, then Extract_BFI.m (you must first inspect hex to set skip_byte_per_capture and skip_byte_per_BFI, which vary by antenna/stream config, capture software, and OS).

Recommendations (staged plan with benchmarks)

Stage 0 — Prove BFI capture (1–2 weeks, ~$50). Buy an MT7612U adapter (Alfa AWUS036ACM, or a generic MT7612U). Deploy your own 802.11ac AP + a client + iperf3 traffic to trigger sounding. Run the capture pipeline above and Wi-BFI main.py. Benchmark: you can reliably reconstruct a BFI angle time-series for a known client MAC, and see VHT compressed-beamforming action frames in Wireshark. If you see no beamforming action frames: verify the AP has beamforming/MU-MIMO enabled, generate more downstream traffic, and confirm your adapter is on the exact channel+width.

Stage 1 — Small controlled identification test (2–4 weeks). 5–10 volunteers, one fixed perspective (across-LOS, hip height), fixed walking path, normal walking only. Collect ~20–40 walks each. Train the LSTM + 2×FC + softmax model in PyTorch (80:20 split, 5 repeats). Benchmark: >90% closed-set accuracy on held-out walks of the same people from the same perspective. Run the empty-room anti-spoofing test (train on people, test on empty-room recordings — top-2 accuracy should be near chance, not matching people). If accuracy <80%: fix antenna height/geometry and increase sounding rate before scaling.

Stage 2 — Integrate with software (2–4 weeks). Stand up FreeRADIUS with SQL (radacct) accounting. Build a fusion service: on a RADIUS Start for user X, open an "expected present" window; use the BFI classifier's confidence (restricted to the closed set of currently-logged-in users) to upgrade attendance from "device-only" to "confirmed present." Log both signals and the confidence score.

Stage 3 — Scale and stress-test (ongoing). Grow to 30+ people; test cross-day (environmental drift), clothing changes, and the multi-person failure mode explicitly. Benchmark honestly: report top-1/top-5 accuracy, FAR/FRR, empty-room rejection, and a degradation curve vs. number of simultaneous movers.

Thresholds that change the plan:

- If multi-person accuracy collapses (likely), restrict the product to single-person choke-points (doorways, turnstiles) rather than open offices — this also fixes the perspective-geometry problem.

- If you require 160 MHz / 6 GHz fidelity to hit the paper's accuracy, you must switch to the expensive route: Intel AX210 NIC(s) + a WiFi 6E AP (e.g., TP-Link Archer BE800). Budget for this only if Stage 1/3 shows the 80 MHz feature count is the binding constraint.

- If cross-day accuracy drops sharply, build periodic re-enrollment into the product from the start.

Caveats

- BFId is closed-set identification (softmax over known people); it does not demonstrate open-set identification of strangers. The dataset is available to researchers on request (ps.kastel.kit.edu/bfid-dataset).

- RTL8812AU corruption evidence is strongest for high-rate multi-stream VHT data frames; BFA action frames are legacy-rate and may partially capture on RTL8812AU, but one direct test reported corrupt VHT feedback — treat MT7612U as the safe cheap choice, and note it maxes out at 2×2 / 80 MHz (no 160 MHz, no 802.11ax).

- The Wi-BFI / BeamSense authors do not publish the exact sniffer NIC; they state "any 802.11ac-compliant NIC in monitor mode." Validate your specific adapter captures uncorrupted VHT compressed-beamforming action frames before investing in data collection.

- Real-world accuracy outside the controlled lab is unproven and expected to be substantially lower than the 99.5% lab figure; the authors themselves flag environmental dynamics, long-term robustness, and cross-network interference as open questions.

- Legal/ethical: this is biometric data (GDPR Art. 9 special category). The KIT study required ethics approval and informed consent. Any real deployment — even a PoC on colleagues — needs explicit consent and a data-protection review.